Name: Hrushikesh Joshi

Roll No: 20ME02002

Results:

- Fixed-Fixed

Enter the value of Modulus of Elasticity of material in Pa: 1

Enter value of area moment of inertia of beam column: 1

Enter length of beam column: 1

First Boundary Condition

Press 1 for Fixed

Press 2 for hinged

Press 3 for guided

Enter the first Boundary condition: 1

Second boundary condition:

Press 1 for Fixed

Press 2 for hinged

Press 3 for guided

Press 4 for Free

Enter the second boundary condition: 1

enter number of terms you want: 10

Matrix R:

[       0,         1, 0, 1]

[       K,         0, 1, 0]

[  sin(K),    cos(K), 1, 1]

[K*cos(K), -K*sin(K), 1, 0]

Determinant of R is

K - 2*K*cos(K) + K*cos(K)^2 + K*sin(K)^2 - K^2*sin(K)

The first n positive roots are:

6.2832    8.9868   12.5664   15.4505   18.8496   21.8082   25.1327   28.1324   31.4159   34.4415

The admissible Function is:

a1*x^2*(x - 1)^2 + a2*x^3*(x - 1)^2 + a3*x^4*(x - 1)^2 + a4*x^5*(x - 1)^2 + a5*x^6*(x - 1)^2 + a6*x^7*(x - 1)^2 + a7*x^8*(x - 1)^2 + a8*x^9*(x - 1)^2 + a9*x^10*(x - 1)^2 + a10*x^11*(x - 1)^2

The energy method solution

1.0e+03 *

0.0395    0.0808    0.1579    0.2387    0.3567    0.4795    0.7267    0.9525    2.5692    3.4052

The eigenvalue solution

1.0e+03 *

0.0395    0.0808    0.1579    0.2387    0.3553    0.4756    0.6317    0.7914    0.9870    1.1862

the error percentage

-0.0000   -0.0000   -0.0002   -0.0014   -0.3790   -0.8119  -13.0848  -16.9083  -61.5849  -65.1646

- Fixed-Free

Enter the value of Modulus of Elasticity of material in Pa: 1

Enter value of area moment of inertia of beam column: 1

Enter length of beam column: 1

First Boundary Condition

Press 1 for Fixed

Press 2 for hinged

Press 3 for guided

Enter the first Boundary condition: 1

Second boundary condition:

Press 1 for Fixed

Press 2 for hinged

Press 3 for guided

Press 4 for Free

Enter the second boundary condition: 2

enter number of terms you want: 10

Matrix R:

[          0,           1, 0, 1]

[          K,           0, 1, 0]

[     sin(K),      cos(K), 1, 1]

[-K^2*sin(K), -K^2*cos(K), 0, 0]

Determinant of R is

K^2*(sin(K) - K*cos(K))

The first n positive roots are:

4.4934    7.7253   10.9041   14.0662   17.2208   20.3713   23.5195   26.6661   29.8116   32.9564

The admissible Function is:

a1*x^2*(x - 1) + a2*x^3*(x - 1) + a3*x^4*(x - 1) + a4*x^5*(x - 1) + a5*x^6*(x - 1) + a6*x^7*(x - 1) + a7*x^8*(x - 1) + a8*x^9*(x - 1) + a9*x^10*(x - 1) + a10*x^11*(x - 1)

The energy method solution

1.0e+03 *

0.0202    0.0597    0.1189    0.1979    0.2974    0.4262    0.6219    1.0316    2.1707    8.4955

The eigenvalue solution

1.0e+03 *

0.0202    0.0597    0.1189    0.1979    0.2966    0.4150    0.5532    0.7111    0.8887    1.0861

the error percentage

-0.0000   -0.0000   -0.0001   -0.0118   -0.2730   -2.6195  -11.0590  -31.0719  -59.0586  -87.2153

- Fixed Guided

Enter the value of Modulus of Elasticity of material in Pa: 1

Enter value of area moment of inertia of beam column: 1

Enter length of beam column: 1

First Boundary Condition

Press 1 for Fixed

Press 2 for hinged

Press 3 for guided

Enter the first Boundary condition: 1

Second boundary condition:

Press 1 for Fixed

Press 2 for hinged

Press 3 for guided

Press 4 for Free

Enter the second boundary condition: 3

enter number of terms you want: 10

Matrix R:

[       0,         1,   0, 1]

[       K,         0,   1, 0]

[K*cos(K), -K*sin(K),   1, 0]

[       0,         0, K^2, 0]

Determinant of R is

K^4*sin(K)

The first n positive roots are:

3.1416    6.2832    9.4248   12.5664   15.7080   18.8496   21.9911   25.1327   28.2743   31.4159

The admissible Function is:

a2*(cos(pi*x) - 1)^2 - a3*(cos(pi*x) - 1)^3 + a4*(cos(pi*x) - 1)^4 - a5*(cos(pi*x) - 1)^5 + a6*(cos(pi*x) - 1)^6 - a7*(cos(pi*x) - 1)^7 + a8*(cos(pi*x) - 1)^8 - a9*(cos(pi*x) - 1)^9 + a10*(cos(pi*x) - 1)^10 - a1*(cos(pi*x) - 1)

The energy method solution

9.8696   39.4784   88.8264  157.9137  246.7401  355.3058  483.6106  631.6547  799.4380  986.9604

The eigenvalue solution

9.8696   39.4784   88.8264  157.9137  246.7401  355.3058  483.6106  631.6547  799.4380  986.9604

the error percentage

1.0e-13 *

0         0         0         0    0.2304    0.3200    0.3526    0.1800    0.2844    0.2304

The code works in a generalized way as seen above. First, the eigenvalue problem is solved then the problem is solved using energy method for n terms, where n is specified by the user.

Based on the comparison of results, accurate values are obtained initially, then the error blows up. The greater the number of terms is demanded by the user the more degree of accuracy is achieved. trigonometric functions as admissible functions yielded better results.

The user can input any number of terms as desired with any combination of valid boundary conditions. In case an invalid boundary condition is input, the user is asked to reinput a valid one.

Enter the value of Modulus of Elasticity of material in Pa: 1

Enter value of area moment of inertia of beam column: 1

Enter length of beam column: 1

First Boundary Condition

Press 1 for Fixed

Press 2 for hinged

Press 3 for guided

Enter the first Boundary condition: 2

Second boundary condition:

Press 1 for Fixed

Press 2 for hinged

Press 3 for guided

Press 4 for Free

Enter the second boundary condition: 4

Invalid Boundary Condition, Please input again

First Boundary Condition

Press 1 for Fixed

Press 2 for hinged

Press 3 for guided

Enter the first Boundary condition:

The code

clear;

clc;

E = input('Enter the value of Modulus of Elasticity of material in Pa: ');

I = input('Enter value of area moment of intertia of beam column: ');

L= input('Enter length of beam column: ');

invalidCondition = true; %flag for the invalid condition

while invalidCondition

flag_1=true;

fprintf('First Boundary Condition \n Press 1 for Fixed \n Press 2 for hinged \n Press 3 for guided \n');

BC1 = input('Enter the first Boundary condition: ');

fprintf('Second boundary condition: \n Press 1 for Fixed \n Press 2 for hinged \n Press 3 for guided \n Press 4 for Free \n');

BC2 = input('Enter the second boundary condition: ');

if (BC1 == 3 || BC1 == 2) && BC2 == 4

fprintf('Invalid Boundary Condition, Please input again\n');

else

invalidCondition = false; % Set the flag to exit the loop

end

end

n=input("enter number of terms you want: ");

%**********************Solves the Eigenvalue problem************************

syms A B C D K x;

l=1;

assume(K~=0);

%charachteristic equation

y(x)= A*sin(K*x) + B*cos(K*x) + C*x + D;

%derivatives up to the third one

dy_dx=diff(y,x);

d2y_dx2=diff(y,x,2);

d3y_dx3=diff(y,x,3);

% BCs according to the applied boundary conditions

if BC1==1 && BC2==1

bc1=y;

bc2=dy_dx;

bc3=y;

bc4=dy_dx;

ig=2*pi;

elseif  BC1==1 && BC2==2

bc1=y;

bc2=dy_dx;

bc3=y;

bc4=d2y_dx2;

ig=4;

elseif  BC1==1 && BC2==3

bc1=y;

bc2=dy_dx;

bc3=dy_dx;

bc4=d3y_dx3+(K^2)*dy_dx;

ig=pi;

elseif BC1==1 && BC2==4

bc1=y;

bc2=dy_dx;

bc3=d2y_dx2;

bc4=d3y_dx3+(K^2)*dy_dx;

ig=pi/2;

elseif BC1==2 && BC2==1

bc1=y;

bc2=d2y_dx2;

bc3=y;

bc4=dy_dx;

ig=2*pi;

elseif BC1==2 && BC2==2

bc1=y;

bc2=d2y_dx2;

bc3=y;

bc4=d2y_dx2;

ig=pi;

elseif BC1==2 && BC2==3

bc1=y;

bc2=d2y_dx2;

bc3=dy_dx;

bc4=d3y_dx3+(K^2)*dy_dx;

ig=pi/2;

elseif BC1==3 && BC2==1

bc1=dy_dx;

bc2=d3y_dx3+(K^2)*dy_dx;

bc3=y;

bc4=dy_dx;

ig=pi;

elseif BC1==3 && BC2==2

bc1=dy_dx;

bc2=d3y_dx3+(K^2)*dy_dx;

bc3=y;

bc4=d2y_dx2;

ig=pi/2;

elseif BC1==3 && BC2==3

bc1=dy_dx;

bc2=d3y_dx3+(K^2)*dy_dx;

bc3=dy_dx;

bc4=d3y_dx3+(K^2)*dy_dx;

ig=0;

end

%making equations out of the boundary conditions

e1=subs(bc1,0)==0;

e2=subs(bc2,x,0)==0;

e3=subs(bc3,x,l)==0;

e4=subs(bc4,x,l)==0;

%Converting the equations into the form Rx=q

eqns=[e1,e2,e3,e4];

vars=[A B C D];

[R,q]=equationsToMatrix(eqns,vars);

% Display the matrix R

disp('Matrix R:');

disp(R);

%calculate the determinant of R

detR=det(R);

disp('Deternminant of R is');

disp(detR);

%convert the matlab equation from symbolic to numeric;

f=matlabFunction(detR);

%find the first n roots of the equation

roots_found = zeros(1, n);

for i = 1:n

if i==1

initial_guess=ig;

else

initial_guess=roots_found(i-1)+pi;

end

% Find the roots one by one starting from an initial guess

roots_found(i) = fzero(f, initial_guess);

end

disp("The first 5 positive roots are:");

disp(roots_found);

P_sol=zeros(1,n);

for i=1:n

P_sol(i)=(roots_found(i))^2*(E*I)/L^2;

end

%*****************************ENERGY METHOD***************************

sym_vars = sym('a', [1 n]); % Creating an array of symbolic variables a1,a2,a3....an;

syms x P;

phi_x=0;

%declaring amissible functions for respective BCs

if BC1==1 && BC2==1

for i=1:n

term=sym_vars(i)*(x^(i+1))*(x-L)^2;

phi_x=phi_x+term;

end

elseif  BC1==1 && BC2==2

for i=1:n

term=sym_vars(i)*(x^(i+1))*(x-L);

phi_x=phi_x+term;

end

elseif  BC1==1 && BC2==3

for i=1:n

%term=sym_vars(i)*(x^(i+1))*(x-3*L/2)^i;

term=sym_vars(i)*(1-cos(pi*x/L))^i;

phi_x=phi_x+term;

end

elseif BC1==1 && BC2==4

for i=1:n

term=sym_vars(i)*(x^(i+1));

phi_x=phi_x+term;

end

elseif BC1==2 && BC2==1

for i=1:n

term=sym_vars(i)*(x)*(x-L)^(i+1);

phi_x=phi_x+term;

end

elseif BC1==2 && BC2==2

for i=1:n

term=sym_vars(i)*(x^(i))*(x-L);

phi_x=phi_x+term;

end

elseif BC1==2 && BC2==3

for i=1:n

term=sym_vars(i)*((x)*(x-2*L))^i;

phi_x=phi_x+term;

end

elseif BC1==3 && BC2==1

for i=1:n

term=sym_vars(i)*(cos(pi*x/L)+1)^i;

phi_x=phi_x+term;

end

elseif BC1==3 && BC2==2

for i=1:n

term=sym_vars(i)*((x-L)*(x+L))^i;

phi_x=phi_x+term;

end

elseif BC1==3 && BC2==3

for i=1:n

term=sym_vars(i)*((x)^(i+1))*(2*x-3*L);

phi_x=phi_x+term;

end

end

disp(phi_x);

dphi_dx=diff(phi_x,x);

d2phi_dx2=diff(phi_x,x,2);

U=((E*I)/2)*int((d2phi_dx2)^2,x,0,L);

W=(P/2)*int((dphi_dx)^2,x,0,L);

PI=U-W; %computing total potential

disp(PI);

eqns = cell(1, n);

for i = 1:n

eqns{i} = diff(PI, sym_vars(i)) == 0;

end

disp(eqns(1));

[V,w]=equationsToMatrix(eqns,sym_vars);

det_V=det(V);

display(det_V);

P_sol_EM=solve(det_V==0,P);

P_num_EM=double(P_sol_EM);

fprintf("The energy method solution \n");

P_num=zeros(1,n);

for i=1:n

P_num(i)=P_num_EM(i);

end

P_num_sort=sort(P_num);

disp(P_num_sort);

fprintf("The eigenvalue solution \n")

disp(P_sol);

error=zeros(1,n);

for i=1:n

error(i)=((P_sol(i)-P_num_sort(i))/(P_num_sort(i)))*100;

end

fprintf("the error percentage \n");

disp(error);